Задание №1

Для указанной топологии необходимо настроить маршрутизацию eBGP для underlay сети DC.

Задание №2*

Настройте ECMP для балансировки трафика в Underlay сети по линкам до разных spine.

Решение:

AS-номера по схеме:

Spine5 — AS 65000 / Spine6 — AS 65000

Leaf1 — AS 65001

Leaf2 — AS 65002

Leaf3 — AS 65003

Leaf4 — AS 65004

Underlay P2P адресация (/31):

| Spine5 e0/3 — Leaf1 e0/1 | 10.0.0.0/31 |
| --- | --- |

| Spine6 e0/3 — Leaf1 e0/0 | 10.0.0.2/31 |
| --- | --- |

| Spine5 e0/2 — Leaf2 e0/1 | 10.0.0.4/31 |
| --- | --- |

| Spine6 e0/2 — Leaf2 e0/0 | 10.0.0.6/31 |
| --- | --- |

| Spine5 e0/1 — Leaf3 e0/1 | 10.0.0.8/31 |
| --- | --- |

| Spine6 e0/1 — Leaf3 e0/0 | 10.0.0.10/31 |
| --- | --- |

| Spine5 e0/0 — Leaf4 e0/1 | 10.0.0.12/31 |
| --- | --- |

| Spine6 e0/0 — Leaf4 e0/0 | 10.0.0.14/31 |
| --- | --- |
| Клиентские сети: Leaf1: 192.168.1.0/24 Leaf3: 192.168.3.0/24 |  |

Конфигурации:

LEAF1 (AS 65001)

hostname Leaf1

interface e0/0
ip address 10.0.0.3 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.1 255.255.255.254
no shutdown

interface e0/2
ip address 192.168.1.1 255.255.255.0
no shutdown

interface e0/3
ip address 192.168.1.2 255.255.255.0
no shutdown

router bgp 65001
bgp log-neighbor-changes

neighbor 10.0.0.0 remote-as 65000
neighbor 10.0.0.2 remote-as 65000

address-family ipv4
network 192.168.1.0 mask 255.255.255.0
neighbor 10.0.0.0 activate
neighbor 10.0.0.2 activate
maximum-paths eibgp 2
exit-address-family

LEAF2 (AS 65002)

hostname Leaf2

interface e0/0
ip address 10.0.0.7 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.5 255.255.255.254
no shutdown

router bgp 65002
bgp log-neighbor-changes

neighbor 10.0.0.4 remote-as 65000
neighbor 10.0.0.6 remote-as 65000

address-family ipv4
neighbor 10.0.0.4 activate
neighbor 10.0.0.6 activate
maximum-paths eibgp 2
exit-address-family

# LEAF3 (AS 65003)

hostname Leaf3

interface e0/0
ip address 10.0.0.11 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.9 255.255.255.254
no shutdown

interface e0/2
ip address 192.168.3.1 255.255.255.0
no shutdown

interface e0/3
ip address 192.168.3.2 255.255.255.0
no shutdown

router bgp 65003
bgp log-neighbor-changes

neighbor 10.0.0.8 remote-as 65000
neighbor 10.0.0.10 remote-as 65000

address-family ipv4
network 192.168.3.0 mask 255.255.255.0
neighbor 10.0.0.8 activate
neighbor 10.0.0.10 activate
maximum-paths eibgp 2
exit-address-family

LEAF4 (AS 65004)

hostname Leaf4

interface e0/0
ip address 10.0.0.15 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.13 255.255.255.254
no shutdown

router bgp 65004
bgp log-neighbor-changes

neighbor 10.0.0.12 remote-as 65000
neighbor 10.0.0.14 remote-as 65000

address-family ipv4
neighbor 10.0.0.12 activate
neighbor 10.0.0.14 activate
maximum-paths eibgp 2
exit-address-family

SPINE5 (AS 65000)

hostname Spine5

interface e0/3
ip address 10.0.0.0 255.255.255.254
no shutdown

interface e0/2
ip address 10.0.0.4 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.8 255.255.255.254
no shutdown

interface e0/0
ip address 10.0.0.12 255.255.255.254
no shutdown

router bgp 65000
bgp log-neighbor-changes

neighbor 10.0.0.1 remote-as 65001
neighbor 10.0.0.5 remote-as 65002
neighbor 10.0.0.9 remote-as 65003
neighbor 10.0.0.13 remote-as 65004

address-family ipv4
neighbor 10.0.0.1 activate
neighbor 10.0.0.5 activate
neighbor 10.0.0.9 activate
neighbor 10.0.0.13 activate
maximum-paths eibgp 2
exit-address-family

SPINE6 (AS 65000)

hostname Spine6

interface e0/3
ip address 10.0.0.2 255.255.255.254
no shutdown

interface e0/2
ip address 10.0.0.6 255.255.255.254
no shutdown

interface e0/1
ip address 10.0.0.10 255.255.255.254
no shutdown

interface e0/0
ip address 10.0.0.14 255.255.255.254
no shutdown

router bgp 65000
bgp log-neighbor-changes

neighbor 10.0.0.3 remote-as 65001
neighbor 10.0.0.7 remote-as 65002
neighbor 10.0.0.11 remote-as 65003
neighbor 10.0.0.15 remote-as 65004

address-family ipv4
neighbor 10.0.0.3 activate
neighbor 10.0.0.7 activate
neighbor 10.0.0.11 activate
neighbor 10.0.0.15 activate
maximum-paths eibgp 2
exit-address-family